PA-10simulator Operation Manual

１. About this program

This is a numerical simulator for PA-10.

PAvision can be used to visualize result of simulation.

By a inputting with a keyboard, you can change a viewpoint and manipulate a virtual PA-10.

２. Instruction

* Please refer to The PROGRAMING MANUAL of PA-10.

URL：（http://www.skillsfestival2007.or.jp/abilympics/en/skillcategories/task/V5%20ProgrammingManual.pdf）

* In this section, the program you have made is called tentatively "pa_src.cpp".

(1)Back up your program and modify a part of the program according to the following instructions.

(2)Put the "include" folder in the folder where "pa_src.cpp" is in.

(3)Modify the header file as follows.

#include <pa.h>	-->	#include "include\pa.h"

#include <paerr.h>	-->	#include "include\paerr.h"

(4)Replace papci.lib with PAsim.lib (which is in the "include" folder)

In the "link", Input "include\PAsim.lib" into the field "Object/library modules".

Note: When you want to operate a real PA-10, re-set "include\PAsim.lib" into "c:\winnt\system32\papci.lib".

(5)Put PAsim.dll into the Debug folder.

３. How to execute your program

ArmX.exe and PAvis.exe:

1) Start up the Wrun(SIM).exe. Select "ARM0","Simulation" and "Select" button.

2) Set the Magnification for simulation at "1".

3) Push "Each axis servo ON" button and select "All".

4) Start up PAvis.exe.

5) In order to manipulate a virtual PA-10, execute your program.

When executing the simulator:

1) If the tip of PA-10 is in front of the canvas, the line will be blue.

2) If the tip of PA-10 is on the canvas, the line will be green.

3) If the tip of PA-10 is behind the canvas, the line will be red.

４. Command from keyboard

Input key(*) :	Behavior

--------------------------------------------------------

w	    : Switch wire frame model to solid model

F	    : Full screen

r / l	    : Viewpoint moves around a virtual PA-10

o / i	    : Zoom out / zoom in

R / L	    : Viewpoint moves right / left

U / D	    : Viewpoint moves up / down

p	    : Viewpoint moves in front of a canvas

X	    : Erase a drawn locus

q	    : Reset all

Esc	    : End

(*) Note: In this program, small letter is distinguished with capital letter.

５. Composition

ErrLog		：The error log of Wrun(SIM).exe is housed.

ARMOFF.ICO	：Icon for Wrun(SIM)

ARMON.ICO	：Icon for Wrun(SIM)

ArmX.exe 	：PA-10simulator (To be started automatically when starting Wrun(SIM).exe )

glut32.dll	：DLL for OpenGL

PAsim.dll	：DLL for PAvis

PAvis.exe	：Program to visualize a result of simulation.

readme.txt	：This file

SIMOFF.ICO	：Icon for Wrun(SIM)

SIMON.ICO	：Icon for Wrun(SIM)

STOP.ICO	：Icon for Wrun(SIM)

Wrun(SIM).exe	：Wrun for simulation